Final Project.

Subject : CSE307 (INTERNETWORKING ESSENTIALS)

Name: Lavanya Puppala

Section: K23UP-(G-2)

Roll No.: 50

Reg.No.: 12300764

Submitted To:

Simarjeet singh malhi

- Physical scenario:

Floor 1,2(MESH TOPOLOGY)

FLOOR 3-5(STAR TOPOLOGY)

FULL SCENARIO:

I have created 5-floor building consisting of different:

- Floors 1–2 have 7 computers each. (use of mesh Topology with switch )

- Floors 3-5 have 7computers each. (use of star Topology with switch)

2. IP VALUES  and Servers:

FLOOR-1

Here I have used VLSM(Variable length subnet masking)so subnet is different its not what basically it is for class A i.e, 255.0.0.0 but here its different due to classless addressing VLSM
floor 1 has :

ip address:10.0.0.2 /28 to 10.0.0.8/28.

subnet mask:255.255.240.0

default gateway: 10.0.0.1

dhcp:10.0.0.9

Router-1

I have done static routing for all the routers which is doing routing manually

Static routing: ip route [destination network] [subnet-mask] [next-hope ip address]

Floor-2 PC ip address(example)

DNS server

Router-2

Floor-2 ip address:10.0.0.18/28 to 10.0.0.24/28

subnet mask:255.255.255.240

default gateway: 10.0.0.17

dns: 10.0.0.25

Floor-3

One PC example

Router-3

HTTP server

Floor-3 ip address:10.0.0.34/28 to 10.0.0.40

subnet mask:255.255.255.240

default gateway: 10.0.0.33

http:10.0.0.41

Floor-4

Floor 4-ip address:172.16.0.2/28-172.16.0.8/28

subnet mask:255.255.255.240

default gateway: 172.16.0.1

common router for floor 4 and floor 5

no server in floor 4

Floor-5

EMAIL

FTP:

Floor-5:

ip address 172.16.0.18/28-172.16.0.24

subnet mask: 255.255.255.240

default gateway:172.16.0.17

ftp:172.16.0.25

email:172.16.0.25

*Communication between pc:

*THANK YOU*